| Borusan EnBW Enerji Istanbul, Turkey |  |  |  |
| --- | --- | --- | --- |
| Saros Hybrid Solar Power Plant Project |  |  |
| [Title] |  |  |
| Doc. No. P0032602-2-1-03 – July 2023 |  |  |  |
| Rev. | 0 |  |  |
| Description | Draft |  |  |
| Prepared by | S. Meral, M. Gash, F. Turan |  |  |
| Controlled by | E. Dogru |  |  |
| Approved by | T. Cinar |  |  |
| Date | July 2023 |  |  |

| Saros Hybrid Solar Power Plant Project [Title] |  |
| --- | --- |

| Rev. | Description | Prepared by | Controlled by | Approved by | Date |
| --- | --- | --- | --- | --- | --- |
| 0 | Draft Issue |  |  |  | July 2023 |
| 1 | Revised based on Client Comments |  |  |  |  |

All rights, including translation, reserved.  No part of this document may be disclosed to any third party,

for purposes other than the original, without written consent of RINA Consulting Mühendislik Limited Şirketi

Table of contents

Page

1	INTRODUCTION	2

1.1	What is The project?	2

1.2	where ıs the project located?	2

1.2.1	Nearest Settlements	2

1.2.2	Other Solar and Wind Power Developments  near Project Area	4

1.3	What are the main components of the Project?	4

1.3.1	What are the Main Project Activities and Schedule?	5

1.4	how wıll materıals for project construction Get To Site?	5

1.5	how does a Solar farm generate electrıcıty?	7

2	PROJECT ratıonale	7

2.1	why ıs The project needed?	7

3	potentıal envıronmental and socıal ımpacts and benefıts of the project	8

3.1	The envıronmental and socıal ımpact assessment process	8

3.2	Who has been consulted and what consultation will be undertaken in the future?	8

3.3	What are the current environmental and social conditions At The Site?	9

3.4	Summary Of Environmental & Social Impacts and Benefits	9

3.5	How Will The Impacts be Managed and Reduced?	15

3.6	How do I find out more about the Project?	15

3.7	How do I have my say?	15

LIST OF TABLES

Table 1-1: Surrounding Settlements	2

Table 1-2: Other Solar Developments in near Project Area	4

Table 3-1: Potential Environmental and Social Impacts and Mitigation Measures	10

LIST OF FIGURES

Figure 1-1: Project Location	3

Figure 1-2: Nearest Settlements around the Project Area	4

Figure 1-3: Site Access to the Cluster 1	6

Figure 1-4: Site Access to the Cluster 2 and Cluster 3	6

Figure 1-5: Scheme of a Solar Farm	7

# INTRODUCTION

Borusan EnBW Enerji Yatırımları ve Üretim A.Ş. (“Borusan”, “the Project Owner”, “the client”) plans to establish and operate Saros Hybrid Solar Power Plant (hereafter “Saros SPP”, “the Project”, “solar farm”) with an installed capacity of 93.9998 MW in an area of 140.62 hectares in Bodurlar, Kocalar, Dedeler and Kovandagi locality of Canakkale province in Turkey. Estimated annual electricity production of the Plant is 551,988,000,00 kWh/year.

An Environmental Impact Assessment (EIA) Report was prepared for the Project and approved by the Ministry of Environment and Urbanisation (MoEU) on April 01, 2022. As Borusan is negotiating process with an Export Credit Agency (the “ECA”) who is considering providing finance for Saros Hybrid SPP, an Environmental and Social Impact Assessment (ESIA) study has been carried out for the Project.

Borusan is committed to demonstrating that it is working to national legislation and international good practice standards. In accordance with this commitment, this Non-Technical Summary (NTS) presents an overview of the key findings of the ESIA. This NTS briefly describes the project, the main findings of the studies undertaken to assess the effects of the planned solar power plant and outlines measures taken to minimise such environmental and social effects.

The purpose of this NTS is to provide information to everyone that may be somehow interested in the Project.

## What is The project?

The project of Saros WPP Auxiliary Resource SPP Project, it is planned to establish 23 Supplementary Source SPPs with a capacity of 93.99 MW on an area of 140.62 ha. The Saros Wind Power Plant, together with the Auxiliary Source Solar Power Plant, will have a total installed capacity of 231,9998 MW.

## where ıs the project located?

The Project is located within the administrative borders of Çan and Merkez Districts of Çanakkale Province. The Project Site covers an area of 140.62 ha. The project location is shown in Figure 1-1.

### Nearest Settlements

The closest settlement to the Project is Dedeler Village followed by Kocalar and Bodurlar Villages. The nearest residential dwelling is located approximately 200m west of Hybrid Solar PV Area – H04 in Dedeler Village. The location of the surrounding settlements in relation to the project are shown in Figure 1-2 and the distances to the project components are presented below.

Table 1-1: Surrounding Settlements

| Settlement | Nearest Project Component | Distance(m) |
| --- | --- | --- |
| Dedeler Village | Hybrid Solar PV Area – H04 | 200 |
| Kocalar Village | Hybrid Solar PV Area – H21 | 380 |
| Bodurlar Village | Hybrid Solar PV Area – H01 | 850 |
| Çanakkale City Center | Hybrid Solar PV Area – H01 | 20,000 |

Figure 1-1: Project Location

Figure 1-2: Nearest Settlements around the Project Area

### Other Solar and Wind Power Developments  near Project Area

xx

Table 1-2: Other Solar Developments in near Project Area

## What are the main components of the Project?

In addition to the installation of PV modules the solar farm also consists supporting infrastructure, typically including:

- Mounting structures (and tracking motors when applicable)

- Cabling

- DC-AC current inverters

- Transformers

- MV & HC Switchgear

- Supervisory Control and Data Acquisition (SCADA) System

- Transmission to grid

- Associated infrastructure and utilities, including:

  - Site security, including fencing and CCTV,

  - Buildings, including onsite substation, connection building, control building, guard cabin, and spare parts storage,

  - Access road and internal road network,

  - Stormwater infrastructure and drainage system.

The preliminary project layout is shown in Figure 1-4.

### What are the Main Project Activities and Schedule?

The construction period of the Project is planned to take 15 months (3 months for vegetative soil stripping works, 8 months for construction works, 8 months for assembly and commissioning, and related activities are expected to continue in parallel). Main activities that will be carried out during each project phase are provided below:

| Project Phase | Works for each phase |
| --- | --- |
| Mobilization | Site Delivery |
|  | Establishment of temporary utility services |
|  | Establishment of living places in facility |
|  | Electrical installation of temporary containers |
| Construction | Land Preparation |
|  | Mounting System Installation |
|  | Panel Installation |
|  | Electrical Installation |
|  | Connection to the System |
| Operation | Connection to YEKDEM |
|  | Connection to the Grid |

## how wıll materıals for project construction Get To Site?

Within the scope of the project, only land arrangement and assembly will take place, and there is no need for any material supply. In case of need, it will be procured from the nearest material quarries, which have all necessary legal permissions, for a fee. The existing and the improved access roads are shown in the map given below (Figure 1-3 and Figure 1-4).

The transfer of the solar panel modules will be arranged with respect to the Routing Plan to be developed. The delivery route for PV panels between the panel factory and the Project site will be determined with respect to Routing Plan. Vehicle access to the is via D210 Çan Çanakkale Road; Figures below show the primary roads that are used to access the site.

• Site access-1 to access the panel areas in the west cluster (Cluster 1) (see Figure 1-3).

• Site access-2 to access the turbines located in the middle cluster (Cluster 2) and north-east cluster (Cluster

3) (see Figure 1-4).

Figure 1-3: Site Access to the Cluster 1

Figure 1-4: Site Access to the Cluster 2 and Cluster 3

## how does a Solar farm generate electrıcıty?

PV modules convert solar radiation directly into electricity through the photovoltaic effect in a silent and clean process that requires no moving parts. The PV effect is a semiconductor effect whereby solar radiation falling onto the semiconductor PV cells generates electron movement. The output from a solar PV cell is DC electricity. A PV power plant contains many cells connected together in modules and many modules connected together in strings to produce the required DC power output.

The workings and general arrangement of a utility scale grid connected solar PV power plant are shown in  Figure 1-5.

Figure 1-5: Scheme of a Solar Farm

# PROJECT ratıonale

## why ıs The project needed?

Solar power is a clean energy source that can be relied on for the long-term future. A solar farm creates reliable, cost-effective, pollution free energy. It is affordable, clean and sustainable. As solar energy is non-polluting and renewable, solar farms create power without using fossil fuels, without producing greenhouse gases or radioactive or toxic waste. There are some potential environmental and social impacts associated with solar farms which are identified in other sections, along with information on the mitigation actions that will reduce the impacts of the Project.

In 2022, electricity consumption in Turkey decreased by 1.2% compared to the previous year, reaching 328.9 TWh, while electricity production decreased by 2.5% compared to the previous year, amounting to 326.2 TWh. In 2022, our electricity production was generated as follows: 34.6% from coal, 22.2% from natural gas, 20.6% from hydroelectric power, 10.8% from wind, 4.7% from solar, 3.3% from geothermal energy, and 3.7% from other sources. As of the end of May 2023, the distribution of our installed capacity by energy sources is as follows: 30.2% hydroelectric energy, 24.2% natural gas, 20.8% coal, 11% wind, 9.6% solar, 1.6% geothermal, and 2.5% other sources.

Turkey has developed and implemented a National Renewable Energy Action Plan to reduce its dependence on energy from external sources and plans to maximize the use of local resources based on the goals to combat climate change. Therefore, Turkey aims to increase the share of renewable energy in the national energy composition and the government has set targets for increasing the use of renewable energy sources. The goal of Turkey for 2023 is increasing the share of total installed power of renewable energy sources more than 30 %; the proposed Project will play a role in achieving this goal.

# potentıal envıronmental and socıal ımpacts and benefıts of the project

## The envıronmental and socıal ımpact assessment process

A national EIA was prepared and “EIA Positive Decision” was obtained from the Ministry of Environment and Urbanisation (MoEU) on April 01, 2022.

As per the IFI and national requirements, the ESIA process was initiated taking into consideration all aspects mentioned within the national EIA with further studies.

A set of generic criteria are used to determine impact significance and are applied across the various environmental and social parameters.

Environmental and social impacts are quantified as much as possible. For cases where quantification is not possible/applicable, a qualitative assessment is conducted using professional judgement, experience and available knowledge, and including the consideration of stakeholder views. Where there are limitations to the data, and/or uncertainties, these are recorded in the relevant chapters, along with any assumptions made during the assessment.

The Project Owner is committed to develop an Environmental and Social Management System (ESMS), which is composed of policies, project assessment documentation, project specific management plans, supporting thematic sub-plans and reporting templates for monitoring progress, to oversee the development, construction, and operational activities of the Project.

## Who has been consulted and what consultation will be undertaken in the future?

In accordance with Article 9 of the Turkish Regulation on Environmental Impact Assessment (latest version published in the Official Gazette dated July 29, 2022, numbered 31907), a Public Consultation Meeting (PCM) was organized in Kocalar village on October 05, 2021 as part of the local EIA studies.

Meeting announcements were published in one national and one local newspaper prior to the meeting date. Besides, meeting invitations were posted in the central areas of Kocalar, Bodurlar, Dedeler, Kovandagı villages and residents of these villages were invited.

Ministry of Environment, Urbanisation and Climate Change General Directorate of EIA, Permission and Inspection, Çanakkale Provincial Directorate of Environment, Urbanization and Climate Change, Boylam Energy Investment Production and Trade. Inc., EN-ÇEV Enerji Çevre Yatırımları ve Danışmanlığı Haritacılık İmar İnş. A.Ş., Mukhtars of the villages and local residents from the nearby villages attended Public Consultation meeting.

At the Public Participation Meeting, detailed information related to the solar power plant was shared with the participants and a visual presentation was made. EN-ÇEV A.Ş. and Boylam Energy Investment Production and Trade. Inc. representatives answered questions asked by the participants and timeline of the Project milestones was delivered also.

## What are the current environmental and social conditions At The Site?

Çanakkale, where the Mediterranean climate is predominant, is in a transition zone between the Mediterranean and Black Sea climates. Terrain elevation, with respect to sea level, increases from the coastal area as you move inland and therefore temperature differences increase. The Project area mainly consists of volcanic and terrestrial clasts.

The nearest surface water to the Project area is 40 km long Sarıçay Stream (Kocaçay) and there are no lakes within the vicinity of the Project area. Sarıçay’s tributary partially runs through the License Area in the west and south clusters. There is Altınzeybek-2 Pond located near Kumarlar Village. Altınzeybek-2 Pond was built in 2019 by a private mining company for providing water supply to the nearby villages for potable use and irrigation.

The baseline air quality (particulate matter and settled dust) and noise level measurements have been undertaken within the project area of influence and found to be compliant with the regulatory limits.

The Saros WF License Area consists of forest land (81.6%), agricultural land (16.5%) and meadows or treasury lands (1.9%). The Project components of the Saros Hybrid SPP will be completely located on agricultural land. he Project Area is composed of Class VI and Class VII soils; that are unsuitable for agriculture.

The settlements that would be potentially affected from the Project are Bodurlar, Dedeler and Kocalar villages. In terms of social baseline studies, data was obtained from the following sources: Secondary data, Quantifiable sample survey (household questionnaire and settlement questionnaire), Depth interview with key informants and Participant observation. Surveys were used to collect quantitative data on a reliable sample of households in the Study Area. The household survey interviews with randomly selected households around the settlements were conducted between November 14 and 15, 2022. According to the survey findings, there are 207 people and approximately 71 households in these villages.

The baseline assessment studies set out that the genders are equally distributed at the villages, 51.6% of the people living in these households are over 50 years old.

Most of the households engage agricultural activities and animal husbandry. The most important source of income is animal husbandry followed by agriculture and pension.  Residents graze their animals in the meadows, agricultural lands or the grasslands near the place where they live. Agricultural activities are limited within the study area due to lack of irrigation sources, lack of young population, low agricultural productivity and high rated consumables (e.g. fuel).

## Summary Of Environmental & Social Impacts and Benefits

The main environmental and social impacts (positive and negative) identified in the ESIA study for the construction and operation phases of the Project and the mitigation measures which will be implemented to remove or reduce the level of impact is presented in Table 3-1.

Table 3-1: Potential Environmental and Social Impacts and Mitigation Measures

Ambient Air Quality

Dust generation comprises the major source of air pollution caused by construction activities especially earthworks. Project earthworks will comprise land levelling and excavation, construction of access roads, excavations for underground cable trenches, ETL towers and substations. Particulate matter is present in the atmosphere for only a short period after release, as particles are heavy enough to settle relatively quickly. Therefore, impacts of dust emission will be localised and will not cause long-term or widespread changes to local air quality. However, deposition of particulate matter will cause short-term impacts on the settlements and agricultural areas in close proximity to the Project area. The following measures will be implemented avoid or minimise the potential impacts on air quality during the land preparation and construction phase of the Project: Daily Visual Checks will be conducted at the selected nearby settlements and necessary mitigations such as stop/pause of construction activities will be taken where required; Dust control methods such as covers, or wind barriers/curtains implemented for open materials storage piles and at locations where dust generating activities will be carried out; Access roads will be chip sealed to prevent dust generation by vehicle movements; Speed limits will be in place for vehicles travelling inside the construction site; Loads in all trucks transporting dust-generating materials will be covered to prevent dust generation; Loading and unloading of materials will be applied without throwing and scattering; Vehicle engines and other machinery will be turned off when not in use, avoiding any unnecessary emissions; Periodical maintenance of machinery and equipment will be carried-out to ensure their good working condition and compliance with standards and technical regulations for the protection of the environment and have appropriate certifications; Minimum number of machinery and equipment will operate at the same time where possible; If any comment related with dust and air quality is received through the Grievance Mechanism, the complaint will be evaluated and where necessary corrective actions will be implemented. During the operation of the plant, electrical energy will be used for heating of the administrative building therefore no emissions will occur, and also vehicle traffic will be minimal. Therefore, the operation of the plant is not anticipated to cause air emissions and have any adverse impact on local air quality. Accordingly, the anticipated impact on local ambient air quality during operations will be negligible.

Noise Impacts

Operation of construction machinery and equipment will generate noise during land preparation and construction activities. Consequently, noise impacts on receptors will occur. The receptors that are likely to be affected by noise impacts are residents of Bodurlar, Kocalar and Dedeler Villages. Cumulative Noise Level at the Source is calculated based on the assumption that the maximum number of all machines/equipment operates at the same time at one location with maximum sound levels in order to demonstrate the worst-case situation. The following mitigations and monitoring activities will be implemented in order to minimise and monitor the potential noise impacts on the sensitive receptors during land preparation and construction phase: Project specific Construction Noise Management Plan will be developed and implemented; Construction activities will be carried out between 08:00 and 18:00 only; Construction vehicle engines and other machinery will be turned off when not in use, avoiding any unnecessary noise generation; When piling works are conducted other machinery will be turned-off to avoid exceedances at the nearest noise sensitive receptors; Periodical maintenance of machinery and equipment will be carried out to ensure their good working conditions; The number of machinery and equipment operating at the same time will be kept minimal where possible; Grievance mechanism will be implemented in line with the Project Stakeholder Engagement Plan. If any grievance related with noise is received from the Noise Sensitive Receptors (NSR) identified in this ESIA, one time noise monitoring will be conducted at these receptors and corrective actions taken where necessary Noise sources during the operation of solar farms are very limited; transformers and inverters will be enclosed and there will be minimal noise emissions from traffic caused by employee transportation. Noise emissions will therefore be negligible. The magnitude of the noise impact during decommissioning phase is anticipated to be similar to the construction noise for a shorter period. Mitigation measures will be similar to the ones taken during the construction.

Land Use, Soil and Visual Impacts

The major Project impacts and/or risks on soils and geology during the land preparation and construction phase include loss of topsoil (in terms of quantity and/or vegetative quality); soil disturbance and erosion, due to earthworks; and soil contamination risk from accidents and improper management of hazardous materials and waste. Mitigation measures to minimise soil loss and erosion will be implemented. Soil contamination will be avoided / minimised through the implementation of a Waste Management Plan, and ensuring the appropriate management of wastes and hazardous materials, including a training programme for Project workers. Following implementation of these mitigation measures, residual impact during land preparation and construction will be negligible. Further impacts on soils as a result of the Project during its operational phase will be minimal, provided proposed revegetation and standard pollution prevention measures are implemented. Visual effects of PV plants arise from changes in the composition and character of views available to receptors affected by the proposed development (e.g. residents, recreational users, tourists etc.). Considering topographical characteristics of the region and layout of the Project, receptors of visual impacts are identified as residents of Dedeler (Moderate significant), Bodurlar (Minor significance) and Kocalar (Negligible significance) villages.

Water and Wastewater

Drinking water will be purchased as bottled water from the local market and utility water will be supplied via tankers from the nearby villages where the standards for utility purposes are met. Estimated water demand for dust suppression is 40 tons. Water for dust suppression will be supplied from the common water source (village fountain) of Etili Village. Water trucks and sprinklers will carry on dust suppression (maximum of 80 tonnes/day) during dry periods. No impact of construction works on water quality of nearby surface waters is anticipated due to the long distance between the Project Site and the nearest surface water bodies. In addition, storage areas (within the laydown area) and construction activities will not be near the watercourses. Therefore, such impacts are not likely to occur. Water demand during operation phase will be mainly for wet panel cleaning. According to the information gathered from the Project Company 470 tonnes/year water will be utilised annually. Based on planned panel cleaning configuration, the impact on availability of the water sources is considered to be minor. However, it is recommended to further assess availability of the selected source based on current capacity and demand once the source is selected.

Waste Management

Main waste types that are generated during the land preparation and construction phase include domestic waste, packaging waste, excavation and construction waste, hazardous waste, and other special hazardous wastes such as medical waste, waste electric/electronic equipment, waste batteries and accumulators, waste oils, waste vegetable oils, end-of-life tires and vehicles, etc. The Project Company and the contractors will be dedicated to avoid and/or minimize impacts due to waste generation by complying with the requirements of Project Waste Management Plan and national legislation as well as applying international standards on waste management. domestic waste generation is estimated to be 74kg/day during construction phase. Additional load to the local landfill facilities due to the Project construction activities is estimated to be 0.39% which is negligible.The daily packaging waste generation is estimated at approximately 22.2 kg during construction period. Generated domestic waste is/will be stored at waste storage areas on site and regularly transferred to the Çanakkale Sanitary Landfill via trucks in line with the local regulations and international standards. Similarly, packaging waste (and all other recyclable wastes) will be segregated and stored at the temporary storage facilities on site and regularly collected by a licensed recycling company for proper management in compliance with the local regulations. Other types of construction specific waste include timber, shavings, wooden pallets, metal scraps and cement bags etc. Materials contaminated with hazardous materials (i.e. fuels, chemicals, paints, oils, solvents et.), waste oils, waste vegetable oils, waste tires, used batteries and accumulator, electronic waste, fluorescents and trace amount of medical wastes are generated during land preparation and construction. Segregation and temporary storage of these wastes will be carried out in line with requirements of Project Waste Management Plan and regularly collected by a licensed waste management/recycling company. Thus, no significant impact is anticipated. Number of project personnel will significantly decrease once the plant is fully operational leading to reduced amount of domestic waste. Similar to the construction phase, domestic waste will be temporarily stored and regularly disposed. During operation phase, there will be limited quantities of hazardous wastes potentially including materials contaminated with hazardous substances, waste oils, waste vegetable oils, waste tires, used batteries and accumulator, electronic waste, fluorescents. A licensed recycling company will regularly collect packaging waste. All, waste management practices will be carried out in compliance with the Project Waste Management Plan and national regulations. Therefore, potential impact from waste management will be negligible If not managed properly, end-of-life and broken/damaged solar panels can cause negative impacts on environment and human health due to their hazardous material content. The Project Company will evaluate potential management strategies and liaise with the PV panel manufacturer or assign a certified waste management company to secure an appropriate and effective recycling/re-use mechanism for end-of-life and broken or damaged solar panels. At Project decommissioning phase, end-of-life PV panels will be the main concern. As with panels broken / damaged during the operational phase, an appropriate recycling/re-use mechanism will be developed for end-of-life solar panels in cooperation with the PV panel manufacturer. If this option is found inapplicable, other recycling facilities will be engaged for this purpose.

Biodiversity

Protected Areas

There are two protected areas: Biga Dağları (Mountains) KBA and Canakkale Strait IBA. Neither of these protected areas overlap with the SPP project area, however the KBA does overlap with the wider WPP site. Although quite limited, information on Biga Dağları KBA suggests that the area was appointed as a KBA due to presence of the three endemic flora species, as it had also been previously identified as an Important Plant Area (IPA), with overlapping boundaries. The two KBA-listed bird species, on the other hand, are Habitats Directive Annex II species due to their declining populations in Europe, but based on expert judgement their populations in the area and also wider region at their preferred habitats are healthy with no direct threats. Canakkale Straight IBA lies 20km west of the Saros SPP and the area was appointed due to presence of a passage movement of Yelkouan Shearwater (Puffinus yelkouan) during the last assessment of the area in 2016. Yelkouan Shearwater are a vulnerable species in the IUCN Red List. The Project Site is not located within or overlaps with any of Turkey’s legally Protected Areas. No direct or indirect impacts of Project activities are anticipated on any protected sites due to distance. The Canakkale Strait IBA is not expected to be impacted by the project due to the distance between the SPP and the IBA.

Habitats and Flora

Habitats within the SPP boundary include: Mixed Deciduous Oak Forests (G1.7), Pinus nigra and Deciduous Mixed Oak Forest (G4.B), East Gariks (F6.2), Permanent Mesotrophic Grassland (E2.1), and Small-Scale Agricultural Lands (I1.2). These habitats were not priority habitats and were primarily modified, therfore they were considered to have negligable conservation value and were not considered individually. The most singifiant impact to the habitats and flora would be habitat loss, fragmentation and degredation as a result of internal road construction and buildings. These impacts are however likely to be limited due to the extent of the permanent habitat loss. Temporary impacts to habitats and flora are also likely as a result of the construction of the solar units. In terms of mitigation all construction and operational working areas will be kept to a minimum to reduce habitat loss and degradation. Off-road access will be prohibited. Plans will be implemented to minimise all construction traffic activities. Dust suppression measures will be implemented, where necessary, during the working periods. All woodland and trees that require further clearance will be supervised by a qualified ecologist. A BMP will be produced for the site to provide further mitigation guidance. A seed collection exercise has been previously completed for the Saros WPP which have been sent to a national seed bank and will be spread across the site post construction. Trees that have been, or are to be, removed will either be replaced through a re-planting scheme within the boundary of the application site or a contribution will be made to the relevant authority which will ensure that the project results in no net loss of woodland / trees and potentially will result in a net gain of this habitat. Vegetation surveys will be completed as part of the operational monitoring, the results of which will be compared to the results of surveys completed for the ESIA. This will monitor the regrowth of habitats that are to be lost during the construction of the solar farm as well as monitoring habitat recovery under the solar arrays. Corrective actions will be taken as necessary to ensure that the project results in a net gain of the Annex 1 habitat within the Project area, including the ETL. Following the precautions, the effect of operational or construction activity on habitats and flora is minimised.

Fauna

Fauna on the project site are likely to be affected by construction through habitat loss/degradation, disturbance (presence of people, artificial lighting, dust and noise), injury or mortality due to construction work and increased traffic and temporary habitat fragmentation. Among the mammals, there are 28 species present in the Project area and its immediate surroundings. According to the IUCN criteria, there are two mammal species (Brown Bear (VU), and Roach's Mouse-tailed Dormouse (VU)) in the threatened categories in the area. Roach’s Mouse-Tailed Dormouse were not noted during the surveys, however they are expected to be present and use hollow trees for nesting. Brown Bear have been previously noted by villagers and may suffer from habitat destruction and fragmentation but were not recorded during the surveys. Bat species are also likely to be affected by habitat loss and degradation, disturbance and injury and mortality. Additionally, loss of breeding and roosting sites could be a significant impact. According to the results of the sonogram analysis of the bat sound recordings, a total of 14 bat species were recorded during the surveys of the Saros SPP in autumn 2022. Four of these species were seen to have a Medium conservation value, due to the loss of habitat. To ensure a minimal impact on the mammals within the site, documents will be produced detailing dust suppression, noise management, ecologist supervision of habitat removal and control of artificial lighting. Common Tortoise was seen regularly throughout the survey period. Therefore, it is selected as one of the indicator species of this study. A translocation of tortoise will be undertaken to ensure no net-loss of this species. Details of the methodology for this translocation will be provided in the BMP. Site staff will be asked to record mammal, amphibian or reptile fatalities on the site roads and this information will be passed on to the Project Ecologist. Corrective actions including signage, decreasing of site wide speed limits or if necessary, construction of animal tunnels will be considered. Monitoring of faunal species will be undertaken during the operational phase of the SPP.

Birds

36 bird species were observed at the Auxiliary Solar Power Plant Project sites. Two species of global conservation concern were recorded and these were; European Turtle-dove (Streptopelia turtur) which is included on the IUCN Red List as Vulnerable and Egyptian Vulture (Neophron percnopterus) which is listed as Near Threatened. No large-scale migration of Egyptian Vulture was observed during the ornithological surveys. Furthermore, the Project Site is not located on the primary migration routes of soaring migrant birds; however, transit passing Egyptian vulture through the Project Site was observed. European Turtle-dove is summer visitor in the AoI and it was usually observed near the villages and agricultural areas during the field studies. Several individuals were also observed within the Project Site and is likely to breed within the woodland areas as well as using the agricultural areas for foraging. According to field study findings, the project site is not a critical nesting/breeding ground for any Endangered avifauna species. Bird species are likely to be affected by loss/degradation of breeding/feeding habitat and disturbance (presence of people, artificial lighting, dust and noise) during construction. Construction impacts will be confined to Project Site and ETL routes including a 500m buffer. To minimise the potential impact to all breeding bird species, vegetation clearance will be undertaken outside of the main bird breeding period if possible. Where this is not possible, the areas to be cleared will be checked for breeding birds prior to the clearance and if nesting birds are found, appropriate mitigation measures will be implemented. This may involve avoiding construction within 50m of the active nest until the chicks have fledged. Further measures, such as the management plan for disturbance, will be noted in the BMP. Monitoring and breeding bird checks will be undertaken during the operational phase of the SPP. Following these mitigation measures, impacts on avifauna will be minimised.

Land Acquisition

A total of 1,406,193 m2 land will be affected by land acquisition for the Project. This area is classified as agricultural land and privately owned. The area is not suitable for agricultural activities and is not used by any people for economic purposes. The land acquisition process is still on-going. Slightly less than 1,000,000 m2 land has been purchased through mutual agreement to date while 411,000 m2 land is subject to land expropriation. Although no one would be required to physically move from their homes or villages as a result of the project, some households would lose the ownership of land. The Project Company aims to undertake the project with compliance with IFC, Performance Standard 5, Land Acquisition and Involuntary Resettlement, as well as with Turkish law on land expropriation. The Project Company recognizes that mutual agreement is the optimal option for land expropriation.

Employment

Limited number of temporary jobs will be created locally during the construction phase. The Project will create approximately 225 (maximum 350 at the peak time) employment opportunities including skilled and unskilled labour. The Project is expected to require approximately five administrative and technical professionals during operation leading to limited job opportunity which will have limited impact on the local communities. The Company will develop a Labour and Employment Management Plan. In line with IFC Performance Standards, The Company will ensure the following during project execution: - Recruitment / hiring of workers from the local communities as much as possible . - Transparency of recruitment / hiring procedure. - Promotion of gender equality during recruitment selection process. - If possible, training and skills development programmes for unemployed people in the project area.

Business Opportunities

The project will create business opportunities for companies at the national and regional level, and also to some extent for companies in the project region. The construction phase also will stimulate the economic activity of the adjacent communities near the solar energy project and the services that these communities provide. The project might create business opportunities for local economy on construction and operation phase. However, those will be very limited. In order to enhance business opportunities related with both phase the following mitigation should be implemented: - Identify opportunities to procure of goods and services from the local area. - Develop a database of local companies and inform the tender process.

Worker Accommodation

Where possible, the EPC will house workers in existing accommodation in Çanakkale City Centre and where using local labour, these workers will be able to reside in their own houses during the construction activities. Where existing accommodation is not an option, the EPC will use temporary camps for the workers

Cultural Heritage

There is no cultural asset within the project area. It is however noted that immovable cultural heritage may be located within rural settlements and hilly mountainous regions. Since the Project’s land preparation and construction phase will involve earthworks and excavation activities, a chance find procedure will be implemented in order to prevent potential harm to any other undiscovered archaeological finds that might be present at the Project Area. The Project will comply with the requirements of Turkish Law with regard to management of any probable chance finds that may be discovered during the Project works.

## How Will The Impacts be Managed and Reduced?

Borusan Enbw has developed a corporate ESMS framework which is covering all Power Projects that are owned by Borusan and its SPV companies. This ESMS framework consists policies and sub-thematic plans that cover all projects. The Project Company will implement this environmental and social management system (ESMS) to oversee the development, construction, and operational activities of the Saros Hybrid SPP Project by tailoring the existing corporate ESMS and extending the Saros WF Project ESMS. The ESMS will be supported by the management plans to be prepared by the Contractor outlining procedures for implementing the requirements of Borusan-EnBW’s requirements. The ESMS framework will be aligned with the requirements of ISO14001:2015 Environmental management, ISO 26000:2010 Social responsibility; and ISO 45001 Occupational Health and Safety management.

## How do I find out more about the Project?

A Stakeholder Engagement Plan (SEP) has been developed for the Project to guide disclosure of public information and stakeholder engagement for the Project. The SEP will be reviewed and updated on a regular basis and updated as necessary. The SEP includes the following:

- Identification of stakeholders and other affected parties

- Public consultations and information disclosure requirements

- Stakeholder engagement programme including methods of engagement and resources

- A grievance mechanism

## How do I have my say?

All concerns and requests can be submitted by filling the grievance form given in Appendix A and sent to the company on the website as given below.

Website				: https://www.borusanenbw.com.tr

Appendix A

Grievance Form

Doc. No. P0032602-2-1-03 – July 2023

RINA Consulting Mühendislik Limited Şirketi Değirmen Sok. Nida Kule İş Merkezi No:18 Kat:9 Kozyatağı 34742 - Istanbul-Türkiye ¦ P. +90 216 3475666 F. +90 216 5509865 info-turkey@rina.org ¦ www.rinaconsulting.org

*****